# MINIMUM SPECIFICATIONS FOR 80201-AHBI AIR COMPRESSOR

Quantity of one (1) Boss Model 80201-AHBI modular above deck hydraulic drive air compressor (s) to include the following as standard equipment:

- All components are frame mounted, plumbed, and wired.  The entire unit is shipped ready for mounting.

- Boss oil flooded, rotary screw air-end designed to produce up to 185 CFM @ 110 PSIG continuously.

- Overall Dimensions:  26”W x 31 1/8”H x 35 1/4”L

- Weight (dry):  700 Lbs

| Delivery @ 110 psig | CFM | 85 | 100 | 125 | 160 | 185 |
| --- | --- | --- | --- | --- | --- | --- |
| Flow @ 2200 psig | GPM | 20.5 | 24.4 | 29.4 | 38.0 | 43.8 |

*Calculations performed @ 85% efficiency mechanical and 96% efficiency volumetric.

- Gear set incorporated into air-end.

- Two-year warranty on the air-end.

- Integrated receiver tank with filler port and drain valve.  External mounted tank not acceptable.

- Self-contained remote mount separator element with easy access spin-on housing.  No exceptions.

- Hydraulic motor coupled to air-end.

- Remote mount, self-contained designed cooling package with automatic thermal sensing control to ensure proper operating temperatures.

- Replaceable spin-on compressor lubricant filter with safety bypass and 25 micron rating.

- Two-stage, dry-type air intake filter with replaceable filter and gravity evacuator.

- Instrumentation to include:  hourmeter, compressor lubricant temperature gauge, and air pressure gauge.

- Safety devices to include:  minimum pressure device, compressor lube hi-temp and hi-pressure safety system, automatic blowdown device, high-pressure relief valve, and necessary warning and information decals.

- Standard 14 gauge polished aluminum canopy with easy access front panel.

- Operation, parts and maintenance manual.

- Installation by factory authorized and trained distributor.

- Standard rear bulkhead connections.

- Optional hose reel feed and bracket.

## INSTALLATION OF AIR STORAGE TANKS

This rotary screw air compressor system is designed to provide continuous volume and constant pressure without the use of an air storage tank.  Within a few seconds of engagement, the compressor will produce full volume and pressure needed to run corresponding air operated equipment without waiting for air tanks to fill.

Air storage tanks are not utilized as standard equipment on vehicle mounted rotary screw compressors.  The installation of air storage tanks, if not done properly, will hinder the performance of the rotary screw compressor.  Boss Industries Inc. strongly discourages the use of an air storage tank as standard equipment for vehicle mounted rotary screw air compressors.

In the rare event air usage is required without operating the vehicle, an air storage tank can be added to your system.  When adding an air storage tank, be sure to follow all state and federal DOT regulations regarding air storage tank usage.