MINIMUM SPECIFICATIONS FOR BOSS BA440 RECIPROCATING AIR COMPRESSOR

TYPE

Boss BA 440 above deck is a hydraulically driven reciprocating air compressor rated at 40 CFM up to 100 PSI. BA440 system is to run intermittently.

COMPRESSOR

BOSS single stage, air cooled, 4-cylinder, pressure lubricated reciprocating air compressor.  Aluminum compressor assembly for lighter weight and higher thermal efficiency. No cast iron compressor assembly acceptable.

| CFM/PSI | RPM | GPM | HYD PSI |
| --- | --- | --- | --- |
| 30/100 (150 max) | 1,150 | 8 | 1,850 (2,100 max) |
| 35/100 (150 max) | 1,400 | 10 | 1,850 (2,100 max) |
| 40/100 (150 max) | 1,650 | 12 | 1,850 (2,100 max) |

Integrated hydraulic system cooler to handle air compressor GPM flow. Compressor to be pressure lubricated via oil pump.  Splash lubrication only is not acceptable. Integrated solenoid and relief valve incorporated into the compressor package.

DRIVE SYSTEM

Direct coupled, gear-type hydraulic motor to compressor input shaft.  
No belt drive allowed

WEIGHT

Compressor installed weight not to exceed 180 lbs.

DEMENSIONS

26 ½”L x 19 ⅛”H x 19 ¾”W

AIR FEED

200 psi 1/2” – 3/4” air feed down stream of the BA 440 air out is required.

AIR RESERVOIR

Minimum 22 Gallon Air Reservoir required.

SERVICE

Routine maintenance items accessible without removal of canopy.

FINISH

Enclosure to be durable, light weight, powder-coat painted and rust resistant aluminum.  Galvanneal sheet metal not acceptable.

COOLING

12VDC electric 14” fan assembly integrated into package for compressor head cooling and single pass hydraulic system cooler. Cooling air intake must not see air temperatures above ambient. Cooling air discharge must have 10” clearance from any obstructions. Ambient temperature for operation should be below 100°F (38°C). The machine may experience high temperature shutdown above this level.

OPERATING SLOPE

20 deg. Maximum operating slope in any direction.

MANUAL

To include operation, maintenance and warranty information.

WARRANTY

18 months. See warranty statement.